Charger Testing Report

Model: 72V 10A
Input Specifications: AC 180-240V, 50/60Hz
Output Specifications: 87.6V, 10A
Production Date: August 2023

- Revision Number:
Rev. 1.0

- Date of Creation or Update:
23/1/2025

- Prepared By:

Muhammad Sabahat Akhter

### Reviewed By:

### Purpose of Testing

The testing was conducted on 15 chargers procured for the demo charging station deployment for Orko. The chargers were selected due to their easy availability. The objective of the testing was to evaluate their performance and ensure they meet the required charging specifications.

### Testing Overview

- Battery Specification:

  - The chargers were tested with 30Ah, 72V battery packs.

- Charging Conditions:

  - Input Voltage Range: 180 - 240V AC 50/60hz

  - Testing Environment: Indoor, ambient temperature of 25°C-30°C.

- Testing Focus:

  - Charging time.

  - Output current consistency.

  - Compatibility with the batteries.

### Observations and Analysis

- Charging Time:

  - All chargers charged the 30Ah batteries in approximately 3 hours and 15 minutes, aligning with expected performance.

- Output Current:

  - The chargers delivered an output current of approximately 9A, slightly below the rated 10A. However, this is within an acceptable range and does not impact performance.

- Performance Stability:

  - No overheating or operational anomalies were observed during the tests. The chargers performed reliably under the specified conditions.

| CHARGER NUMBER | CHARGING TIME | BATTERY CAPACITY | CHARGING CURRNET | REMAKERS |
| --- | --- | --- | --- | --- |
| Charger 1 | 3 H : 16M approx. | 30 A | 8.7A | Within acceptable range and required to change output connectors |
| Charger 2 | 3 H : 17M approx. | 30 A | 8.6A | Within acceptable range and required to change output connectors |
| Charger 3 | 3 H : 16M approx | 30 A | 8.7A | Within acceptable range and required to change output connectors |
| Charger 4 | 3 H : 15M approx | 30 A | 8.8A | Within acceptable range and required to change output connectors |
| Charger 5 | 3 H : 14M approx | 30 A | 8.9A | Within acceptable range and required to change output connectors |
| Charger 6 | 3 H : 10M approx | 30 A | 9A | Within acceptable range and required to change output connectors |
| Charger 7 | 3 H : 15M approx | 30 A | 8.8A | Within acceptable range and required to change output connectors |
| Charger 8 | 3 H : 10M approx | 30 A | 9A | Within acceptable range and required to change output connectors |
| Charger 9 | 3 H : 11M approx | 30 A | 8.9A | Within acceptable range and required to change output connectors |
| Charger 10 | 3 H : 16M approx | 30 A | 8.7A | Within acceptable range and required to change output connectors |
| Charger 11 | 3 H : 6M approx | 30 A | 9.4A | Within acceptable range and required to change output connectors |
| Charger 12 | 3 H : 9M approx | 30 A | 9.1A | Within acceptable range and required to change output connectors |
| Charger 13 | 3 H : 11M approx | 30 A | 8.9A | Within acceptable range and required to change output connectors |
| Charger 14 | 3 H : 15M approx | 30 A | 8.8A | Within acceptable range and required to change output connectors |
| Charger 15 | 3 H : 11M approx | 30 A | 8.9A | Within acceptable range and required to change output connectors |

### Conclusion

- The chargers performed satisfactorily in charging 30Ah battery packs within the expected time frame.

- While the output current was slightly below the rated 10A, it was slightly varying across all units w.r.t Soc and within acceptable limits for demo deployment.

- These chargers are suitable for the demo charging station deployment for Orko, given their availability and consistent performance.