### Operational Guidelines for Swapping Charging Station

- Revision Number:
Rev. 2.0

- Date of Creation or Update:
5/2/2025

- Prepared By:

Muhammad Sabahat Akhter

### Reviewed By:

### 1. Pre-Operation Checks

Before starting any operation, please ensure the following:

- Main Circuit Breaker & Safety Breakers:

  - Ensure that the main circuit breaker (63A) and the safety double breakers are in the "ON" position.

- Power Supply:

  - Verify that a stable 220V AC (50/60 Hz) is supplied through the main circuit breaker to the charging station.

- Wi-Fi Connection:

  - Confirm that the charging station has a stable Wi-Fi connection with strong signal strength. The connection should be within a 10-meter range.

- Charger Connection:

  - Check that all chargers are securely connected and powered.

- Dashboard & Server Console:

  - Ensure the dashboard and server console are operational, displaying accurate data.

- Safety & Environment:

  - Inspect the area around the charging station for cleanliness and dryness. Ensure that it is free of obstructions.

  - Note: Avoid placing the swap station directly under sunlight. It should be placed under a shed to avoid heat-related issues.

### 2. Battery Charging Procedure

Follow these steps to ensure safe and efficient charging:

- Connecting Batteries:

  - Connect one battery at a time to avoid sudden current surges.

- Connector Check:

  - Ensure that the battery connectors are securely plugged into their designated slots.

- Monitor Charging:

  - Regularly monitor the charging status of each battery via the dashboard and server console.

- Unplugging Batteries:

  - Do not unplug a battery while it is actively charging unless safety precautions are taken.

### 3. Troubleshooting Guidelines

In case of any issues, follow the appropriate troubleshooting steps:

- Slot Not Working:

  - Perform a soft refresh of the microcontroller and checkout its hardware connections for the affected slot.

  - Verify communication between the master controller and the affected slave circuit.

- Charger Stops Functioning:

  - Check for visible signs of damage or overheating.

  - Report any issues to the maintenance team for further inspection.

- Server-Side Discrepancies:

  - Cross-check the server console readings.

  - Report any mismatches to the technical team for analysis.

### 4. Full-Load Testing Procedure

To validate the system performance under maximum load conditions:

- Testing All Batteries:

  - Regularly perform full-load testing by connecting all 12 batteries to the charging station.

- Monitor Load:

  - During full-load testing, monitor the AC load and charging times to ensure that all components (chargers, circuit breakers) are operating within their specified limits.

- Document Results:

  - Record all testing results and report any anomalies to the technical team for further analysis.

### 5. Safety Measures

Adhere to the following safety guidelines to prevent hazards:

- Certified Components:

  - Only use certified batteries, chargers, and components.

- Signs of Danger:

  - Immediately stop operation if sparks, overheating, or abnormal noises are observed, and report to the relevant team.

- Ventilation:

  - Ensure proper ventilation around the charging station to prevent overheating.

- Do Not Bypass Safety Mechanisms:

  - Never bypass safety features like circuit breakers or protective wiring.

- Emergency Stop:

  - In the event of an emergency, switch off the main circuit breaker immediately.

### 6. Maintenance Guidelines

Regular maintenance ensures efficient operation:

- Routine Inspections:

  - Schedule periodic inspections of chargers, circuit breakers, and slave circuits to ensure optimal performance.

- Connector Maintenance:

  - Regularly clean and inspect charging connectors and slots for wear or damage.

- Server Functionality:

  - Periodically test the server-side functionality to ensure accurate data display and system control.

- Firmware Updates:

  - Perform firmware updates only after thorough testing to ensure system stability.

### 7. Wi-Fi and Communication

Ensure uninterrupted communication for smooth operation:

- Wi-Fi Range:

  - Ensure that the charging station is within a 10-meter range of strong Wi-Fi signals.

- Wi-Fi Issues:

  - In case of communication disruptions, notify the network team immediately to resolve any connectivity issues.

### 8. Reporting

Document all activities for record-keeping and analysis:

- Log Activities:

  - Log all operational activities, anomalies, and maintenance tasks in a centralized system or report to the operations team.

- Timely Reporting:

  - Ensure any issues related to chargers, slave circuits, or server functionality are reported promptly.

### Peak Load and AC Load Current Specifications

- Peak Load (KW): The peak load of the swappable charging station is around 16 kW.

- AC Load Current: The maximum AC load current is around 71 A.

This operational guideline ensures that the charging station is properly maintained and operated for optimal performance and safety.