### Operational Guidelines for Swapping Charging Station

- Revision Number:
Rev. 1.0

- Date of Creation or Update:
28/1/2025

- Prepared By:

Muhammad Sabahat Akhter

### Reviewed By:

- Pre-Operation Checks

- Ensure the main circuit breaker (63A) and safety double breakers are in the ON position.

- Ensure the Single phase 220V AC 50/60 Hz is supplied through the main circuit breaker.

- Verify the charging station has a stable Wi-Fi connection with strong signal strength (within 10 meters).

- Check that all chargers are securely connected and powered.

- Confirm that the dashboard and server console are operational and displaying accurate data.

- Inspect the area around the charging station to ensure it is clean, dry, and free of obstructions.

- Do not Place the swap station directly under the sunlight. Place it under the Shed.

#### 2. Battery Charging Procedure

- Connect one battery at a time to prevent sudden surges in current.

- Ensure the battery connectors are securely plugged into the designated slots.

- Monitor the charging status of each battery on the dashboard and server console.

- Avoid unplugging a battery while it is actively charging unless necessary precautions are taken.

#### 3. Troubleshooting Guidelines

- If a slot is not working as expected:

  - Perform a soft refresh of the microcontroller for that slot to restore functionality.

  - Verify communication between the master controller and the affected slave circuit.

- If a charger stops functioning:

  - Inspect for any visible signs of damage or anomalies, such as sparks or overheating.

  - Report the issue to the maintenance team for further inspection and repair.

- For server-side discrepancies (e.g., parameters not showing on the dashboard):

  - Cross-check the server console readings and report any mismatches to the technical team.

#### 4. Full-Load Testing Procedure

- Perform regular full-load testing by connecting all 12 batteries to validate system performance under maximum load conditions.

- Monitor AC load and charging times during testing to ensure chargers and circuit breakers are operating within specifications.

- Document the results and report any anomalies for further analysis.

#### 5. Safety Measures

- Use only certified batteries, chargers, and components with the charging station.

- Immediately stop operation and report if sparks, overheating, or abnormal noises are observed.

- Ensure proper ventilation around the charging station to prevent overheating.

- Do not bypass any safety mechanisms such as circuit breakers or protective wiring.

- In case of an emergency, switch off the main circuit breaker immediately.

#### 6. Maintenance Guidelines

- Schedule routine inspections of all chargers, circuit breakers, and slave circuits to ensure optimal performance.

- Regularly clean and inspect the charging connectors and slots for wear or damage.

- Test server-side functionality periodically to maintain accurate data display and control features.

- Perform firmware updates for the microcontroller and server system only after thorough   testing.

#### 7. Wi-Fi and Communication

- Ensure the charging station is placed within a 5-meter range of strong Wi-Fi signals for uninterrupted communication.

- In case of Wi-Fi issues, notify the network team immediately for resolution.

#### 8. Reporting

- Log all operations, anomalies, and maintenance activities in a centralized system or report to the operations team.

- Ensure timely reporting of any issues related to chargers, slave circuits, or server functionality.