JSON DATA FORMAT AND COMMUNICATION FLOW

- Revision Number:
Rev. 3.0

- Date of Creation or Update:
17/1/2025

- Prepared By:

Muhammad Sabahat Akhter

### Reviewed By:

### JSON Data Format Overview

The following is a structured JSON message generated by the ESP8266 d1 mini. This format is designed to transmit data about a battery management system and additional features to the Cloud server via websockets. The key fields provide real-time telemetry of various parameters:

#### Sample JSON Structure:

{

"Slot_ID": 1,

"BRN_ID": "XXYY",  // Battery registration number

"Voltage": 162.5,    // Battery voltage in volts

"Current": 9.2,     // Battery current in amperes

"SoC": 85,          // State of Charge in percentage

"Ext_Temperature": 30.0, // External temperature in Celsius

"Bat_Temperature": 35.5, // Battery temperature in Celsius

"Flame_status": 0,  // 0 = No flame, 1 = Flame detected

"Door_status": 1,   // 0 = Door closed, 1 = Door open

"Charging_status": 1 // 0 = Not charging, 1 = Charging

}

### Dynamic Field Inclusion

The JSON structure is flexible and fields may increase or decrease based on the system features enabled. For example:

- Added Features:

  - Humidity, pressure, or GPS data.

  - Additional battery-related fields such as cycles or capacity.

- Removed Features:

  - If a feature (e.g., flame detection) is not included in the system, its corresponding field may be omitted.

### ESP8266 to STM32 Communication

#### Serial Communication Setup

- The master ESP8266 module communicates with the STM32 using its available TX and RX pins via UART (Universal Asynchronous Receiver-Transmitter).

- The goal is to receive all the parameters using serial communication from stm32 to esp8266 for further processing data to cloud server.

- The STM32 will receive the JSON message string through this UART interface for further processing.

#### Hardware Requirements:

- Connections:

  - ESP8266 TX -> STM32 RX

  - ESP8266 RX -> STM32 TX

  - Common GND between ESP8266 and STM32.

- Baud Rate:

  - Set an appropriate baud rate (e.g., 9600, 115200) for reliable communication.

- Voltage Levels:

  - Ensure the voltage levels are compatible (3.3V logic). Use a level shifter if the STM32 operates at a higher voltage (e.g., 5V).

#### Software Implementation:

- STM32:

  - Configure Uart communication to receive and transmit response with Esp8266

  - Convert the data into a JSON string and transmit it over the UART using Serial.print() or similar commands to esp8266.

  - Example:

  - String message ="Slot_ID: " + String(1) +

  - "BRN_ID: " + String(response.brnID)+

  - "Voltage: " + String(response.voltage) +

  - "Current: " + String(response.current) +

  - "SoC: " + String(response.soc) +

  - "Ext_Temperature " + String(response.ext_Temp) +

  - "Bat_Temperature:"+ String(response.temperature) +

  - "Flame_status: " + String(response.fireDetected) +

  - "Door_status: " + String(response.lockStatus) +

  - "Charging_status:"+ String(response.charging_status);

Serial.println(message);

- ESP8266:

  - Configure UART to receive JSON data.

  - Save the received parameters and transmit to our cloud server for further processing.

CHARGING STATION COMMUNICATION FLOW

This diagram represents a communication and data flow system involving a Daly BMS battery, Master PCB, Slave PCB, and a cloud server. Here's a brief explanation of each component and the protocols used:

### 1. Daly BMS Battery

- Description: The battery management system (BMS) monitors and manages the battery's health, voltage, current, and other parameters.

- Communication:

  - The Slave PCB communicates with the Daly BMS battery via CAN Communication Protocol.

  - CAN (Controller Area Network) is for real-time data exchange in battery systems.

### 2. Slave PCB

- Role:

  - Acts as a bridge between the Daly BMS battery and the Master PCB.

  - Collects battery data such as voltage, state of charge (SoC), temperature, etc., through CAN communication.

  - Collects and control sensors data.

- Communication with Master PCB:

  - Uses the ESP-NOW Protocol to wirelessly send the collected data to the Master PCB.

  - ESP-NOW: A lightweight and fast wireless communication protocol provided by ESP8266/ESP32, ideal for short-range communication with minimal latency.

### 3. Master PCB

- Role:

  - Receives data from the Slave PCB.

  - Processes and relays data to a cloud-based server for remote monitoring and control.

- Communication with Cloud Server:

  - Uses the WebSocket Protocol for communication.

  - WebSocket: Enables full-duplex communication, making it efficient for real-time updates and control.

### 4. Cloud Server

- Role:

  - Acts as a centralized system to monitor all connected devices.

  - Provides a web or mobile interface for users to view battery and system data in real time.

  - Sends control commands (e.g., to open slot door) back to the Master PCB.

### Protocols Used

- CAN Communication: For reliable wired communication between the BMS and the Slave PCB.

- ESP-NOW: For wireless, fast, and lightweight data transfer between the Slave PCB and Master PCB.

- WebSocket Protocol: For real-time, bidirectional communication between the Master PCB and the cloud server.

### Purpose of the System

- Real-time monitoring: Battery parameters are monitored via the cloud.

- Wireless Communication: Simplifies the system by using ESP-NOW for short-range and WebSocket for long-range communication.

- Remote control: Allows users to control and monitor data from anywhere through the cloud.

Single Slave PCB description

Purpose of the Slave PCB
 This flowchart represents a hierarchical communication system where a master microcontroller manages data exchange with multiple slave microcontrollers, with each board containing 4 slave microcontrollers. The purpose of this system is to ensure efficient, reliable, and sequential communication within a multi-microcontroller setup.