### Swappable Charging Station Functionality and Performance Report

- Revision Number:
Rev. 1.0

- Date of Creation or Update:
30/1/2025

- Prepared By:

Muhammad Sabahat Akhter

### Reviewed By:

### Suleman Shahid

### 1. Overview

The swappable charging station consists of 12 slots, each equipped with a 72V 10A charger, all of which are in working condition. The system is designed to monitor and display critical battery parameters for each slot, including:

- Battery Voltage (V)

- Battery Current (A)

- State of Charge (SoC %)

- Slot Temperature (°C)

- Battery Temperature (°C)

- Door Status

- Charging Status

The station operates efficiently after all slots sync with the master controller upon startup. Any unsynced slots are successfully reconnected after performing a hard reset. The system remains stable and functions properly until it is powered off.

### 2. Identified Communication Issue

After extensive testing, a recurring issue has been identified:

- When the charging station is powered off and turned back on, or in the event of a power outage (load shedding), some slave microcontrollers lose communication with the master controller.

- This results in a loss of data synchronization, preventing affected slots from reporting their parameters to the system.

- Affected slave microcontrollers need to be manually refreshed to restore communication and re-establish synchronization with the master controller.

### 3. Investigation & Resolution Efforts

- Multiple tests were conducted to diagnose the root cause of the communication failure.

- The issue is linked to the syncing process between the master and slave microcontrollers following a power cycle.

- Maria has been assigned to resolve this issue and is working on a permanent fix to enhance system stability and user experience.

- The final resolution is expected to be completed by February 3, 2025.

### 4. Next Steps & Recommendations

To mitigate and permanently resolve the communication issue, the following actions are recommended:

- Firmware Enhancement: Implement an improved auto-reconnection mechanism for the slave microcontrollers to prevent desynchronization after a power cycle.

- Soft Refresh Automation: Develop a system for automatic soft refresh of the ESP microcontrollers, eliminating the need for manual intervention.

- Extended Testing: Perform additional validation tests after the fix is applied to ensure long-term system stability.

- Real-Time Monitoring: Continuously observe the system’s performance, especially after power cycles, to confirm that all 12 slots remain consistently connected.

### 5. Conclusion

The swappable charging station is functioning properly under normal conditions. However, the communication desynchronization issue observed after power cycles requires optimization. Maria is actively working on a resolution, which is expected to be completed by February 3, 2025. Once the final fix is implemented, further validation testing will be conducted to ensure a seamless and reliable user experience.